COMPTIA NETWORK+ – COMPLETE ASSIGNMENT

Network Fundamentals, Building Networks, Addressing, Switching, Routing, Wireless & Troubleshooting

Name: Rehan Vora

# 1. CompTIA Network+ – Network Fundamentals and Building Networks

## Network Purpose

A network allows devices to exchange data and share resources. Network engineering combines cabling, switching, routing, wireless, addressing, services, security and monitoring.

## Network Types

LAN: local area network.

WAN: geographically separated networks.

WLAN: wireless LAN.

PAN: personal-area network.

SOHO: small office/home office.

Data-center network: high-density infrastructure for servers and applications.

## Topologies

Star: endpoints connect to a central device.

Mesh: multiple paths provide redundancy.

Bus and ring: important historical/specialized concepts.

Hybrid: combination of topology designs.

# 2. OSI and TCP/IP Models

## OSI Seven Layers

7 Application: services used by applications.

6 Presentation: representation/encoding.

5 Session: session management.

4 Transport: TCP/UDP and ports.

3 Network: IP addressing and routing.

2 Data Link: Ethernet frames, MAC and switching.

1 Physical: signals, media, connectors and bits.

The TCP/IP model groups networking functions into Application, Transport, Internet and Link/Network Access layers. TCP/IP is the architecture used by the Internet.

# 3. Network Devices

Switch: forwards Layer-2 frames using MAC addresses.

Router: forwards Layer-3 packets between networks.

Firewall: enforces security policy.

Access point: bridges wireless clients to a LAN.

Modem/ONT: terminates or converts certain ISP access technologies.

Load balancer: distributes application traffic among servers.

Proxy: intermediary for selected application traffic.

# 4. Ethernet and Cabling

Ethernet is a major LAN technology standardized by IEEE 802.3. Ethernet frames contain source and destination MAC addresses and an error-detection mechanism.

## Copper

Twisted-pair Ethernet is common in structured cabling.

Cat5e, Cat6 and Cat6a are common categories.

Modern devices often support auto-MDI/MDIX, but correct standards and pinouts remain important.

## Fiber

Multimode fiber is common for shorter high-speed links.

Single-mode fiber supports much longer distances.

Fiber is highly resistant to electromagnetic interference.

# 5. IP Addressing and Subnetting

## IPv4

IPv4 uses 32-bit addresses. Private ranges are 10.0.0.0/8, 172.16.0.0/12 and 192.168.0.0/16. A subnet mask/prefix identifies the network portion and the default gateway provides a next hop outside the local subnet.

## IPv6

IPv6 uses 128-bit addresses. Common types include global unicast, link-local and multicast. Link-local addresses use FE80::/10. IPv6 does not use broadcast.

## Subnetting Example

Dividing 192.168.10.0/24 into /26 creates four subnets: 192.168.10.0/26, .64/26, .128/26 and .192/26. Each /26 contains 64 addresses and, in ordinary IPv4 subnetting, 62 usable host addresses.

# 6. Switching Fundamentals

A switch learns source MAC addresses and associates them with incoming ports. It uses its MAC address table to determine where to forward frames.

Frame arrives on a switch port.

Switch learns the source MAC and port.

Switch checks the destination MAC.

Known destination: forward to the associated port.

Unknown destination: flood within the relevant VLAN.

# 7. VLANs and Trunking

A VLAN creates a logical Layer-2 broadcast domain. VLANs segment a switched network into separate logical networks.

Access port: normally carries one VLAN for an endpoint.

Trunk: carries multiple VLANs using IEEE 802.1Q tagging.

Native VLAN: associated with untagged traffic on an 802.1Q trunk, depending on configuration.

# 8. Routing Fundamentals

Routing moves packets between IP networks. Routes can be connected, static or dynamically learned.

Static route: manually configured.

Dynamic routing: learned using routing protocols such as OSPF.

Default route: used when no more-specific route matches.

Longest-prefix match: most specific matching route wins.

# 9. DHCP and DNS

## DHCP

DHCP automatically provides IP address, subnet mask, default gateway and DNS information. The common IPv4 process is DORA: Discover, Offer, Request and Acknowledgment.

## DNS

DNS translates names and stores many record types including A, AAAA, CNAME, MX and TXT.

# 10. Wireless Networking

Wireless LANs use radio frequencies between clients and access points.

2.4 GHz generally has longer range but fewer non-overlapping channel choices and more interference.

5 GHz generally provides more channels and capacity but often less range.

6 GHz is available for supported Wi-Fi 6E/Wi-Fi 7 deployments where regulations permit.

Use strong wireless authentication/encryption such as WPA2 or WPA3 where supported.

# 11. Practical – Build a Small Network

Create two switches and several PCs. Create VLAN 10 and VLAN 20, assign access ports, create an 802.1Q trunk and configure inter-VLAN routing.

## Switch Configuration

vlan 10
 name USERS
vlan 20
 name ADMIN
interface fa0/1
 switchport mode access
 switchport access vlan 10
interface fa0/2
 switchport mode access
 switchport access vlan 20
interface fa0/24
 switchport mode trunk
 switchport trunk allowed vlan 10,20

## Router-on-a-Stick

interface g0/0
 no shutdown
interface g0/0.10
 encapsulation dot1Q 10
 ip address 192.168.10.1 255.255.255.0
interface g0/0.20
 encapsulation dot1Q 20
 ip address 192.168.20.1 255.255.255.0

## Verification

show vlan brief
show interfaces trunk
show ip interface brief
show ip route
ping 192.168.20.10

# 12. Network Troubleshooting

Use a structured approach: identify symptoms, establish a likely cause, test, implement the fix, verify and document.

Check power, cables and link status.

Check VLAN membership and trunking.

Check IP address, subnet mask and gateway.

Check routing and DNS.

Test hop by hop using ping/traceroute.

Verify the final result and document it.

## Useful Cisco Commands

show ip interface brief
show interfaces
show vlan brief
show interfaces trunk
show mac address-table
show ip route
show cdp neighbors
show running-config
ping
traceroute

# 13. Basic Network Security

Use strong authentication and least privilege.

Segment networks using VLANs and security controls.

Use secure wireless encryption.

Maintain device software/firmware.

Disable unused services/ports where appropriate.

Protect management interfaces and monitor/log network activity.

# 14. Practical Evaluation

Identify switches, routers, firewalls and access points.

Explain OSI and TCP/IP models.

Identify copper and fiber concepts.

Create an IPv4 addressing plan.

Perform basic subnetting.

Configure VLANs and access ports.

Configure an 802.1Q trunk.

Explain MAC learning.

Configure and verify basic routing.

Explain DHCP and DNS.

Configure/test a basic WLAN.

Troubleshoot a LAN using show, ping and traceroute.

# 15. Viva Questions

## What is the difference between a switch and router?

A switch primarily forwards Layer-2 frames inside a LAN; a router forwards Layer-3 packets between IP networks.

## What is a VLAN?

A logical Layer-2 broadcast domain used to segment a switched network.

## What is a default gateway?

The Layer-3 next hop used by a host to reach destinations outside its local subnet.

## What is DHCP?

A protocol that automatically supplies IP configuration to clients.

## What is DNS?

A distributed naming system that maps names to network information.

## What is subnetting?

Dividing an IP network into smaller logical networks.

## TCP vs UDP?

TCP provides connection-oriented, reliable, ordered delivery; UDP is connectionless and has lower overhead but does not provide TCP-style reliability.

# 16. Research Note

This report is a study assignment based on standard CompTIA Network+ networking concepts. CompTIA updates certification objectives periodically, so compare this report with the current official CompTIA objectives before an exam.